bkharris(N)			4 term (92 dB) Blackman-harris window. Used just like the built in 					window functions.

cordemo				Demonstration of auto- and cross-correlation on noise, a sinusoid and 					their sum. Can alter the number of point, the freq of the sinusoid, and 					the amplitude scale factor on the noise. This is done in the “m” file by

altering the default variables at the top of the file.

cstring				This is called by other functions. You need it , but don’t need to know 					how to use it.

fresp				Plots the magnitude spectrum of data normalized by data length.

The usage of the function is as follows:

fresp(data) plots the mag response of data using a transform of 						length equal to the length of the data.

freq(data, xformLen) uses a transform of length xform length.

freq(data, xfromLen, 'window') windows the data. Use the 							name of the matlab window function in string form,

i.e. 'blackman' (note the single quotes). If the kasier 							window is used then the parameter beta is required, 							ie. freq(data, xformLen, 'kaiser', beta).

interpdm(data, L)				Interpolation demo. Takes data  and interpolates by L. 						Filtering is done with zero-order hold and sinc interpolator.

[h,e,out]=lmstest(input,  desired, numTaps, mu, numIterations)

This is the lms demo. The  outputs are h (filter coef), e 						(modified error), o (output of the predictor). The zeros are the 						zeros of [ 1 -h’].

y=sine(amp, analog freq, sampling freq, number of points, phase,)

Generate a real sinusoid.

y=mod(signal, digital modulation freq, type)

Modulation of a signal. The type parameter takes a string (use 						single quotes. ‘real’ performs real modulation. ‘complex’ 						performs complex modulation.

[y,n]=msine				Generates signal composed of multiple sine waves. It prompts 						you for the required info. Here, if asked for real or complex,  						do not use the single quotes.

nrfresp					This is a version of fresp called by other functions. You need it 					but do not need to know how to use it.

[y,n]=pulse(amplitude, number of points, index of start of pulse, length of pulse in sample)

This function will generate a pulse. The index of the first point 					is 0.

y=sample(number of points)		A function to explore sampling. It will prompt for an analog 						frequency and a sampling frequency. It plots the sampled sine.

smpdemo			Demo of sampling. It prompts you for info.

y=srexpand(data, L)		Zero extends data with L-1 zeros between each data point.

stats(data)			Generates various parameter of real data.